Palm OS Simulator Manual

Table Of Contents

Palm OS Simulator Setup	3

Hardware Connections	3

Palm OS Simulator Installation	3

Determining the COM Port	4

Changing the COM Port After Install Has Completed	4

Using Palm OS Simulator With JCI Applications	5

Desktop Icons and Windows Menu Items	5

Launching JCI Palm OS Applications	5

Launch Examples	6

Generating JCI Wiring Checkout Reports	7

Requirements	7

Saving A Palm OS Session	7

Updating The Palmsim.ini file	8

Updating The Palm Hot Sync Manager	9

Configuring The Palm OS Simulator For Generating Wiring Checkout Reports	10

Launching the Palm Sync Application	13

Wiring Checkout Report Location	14

Appendix	15

Determining the COM Port	15

Troubleshooting Tips	16

Wiring Checkout Tool  - Cannot Run Simulator After Saving / Restoring Session	16

Wiring Checkout Tool  - Wiring Checkout Hot Sync Does Not Generate Report	16

Wiring Checkout Tool  - Saved Sessions Do Not Get Restored	16

Wiring Checkout Tool – Hot Sync Error When Generating Wiring Checkout Reports	16

# Palm OS Simulator Setup

## Hardware Connections

If your laptop has a COM port, a USB to RS232 converter is not needed.

    - Hardware Connections With Out USB

If your laptop only has USB ports, a USB to RS232 converter is required.  Follow the installation procedure for this device to install the required drivers.

    - Hardware Connections With USB

## Palm OS Simulator Installation

Run the setup.exe and follow the Install Shield procedure.

    - Palm OS Setup Program

### Determining the COM Port

During setup, the install program will automatically determine the COM port from a list of available COM ports on your machine.  Important: If you are planning to use a USB COM port option, install the USB driver and have the USB converted attached to your USB port before beginning the install program, otherwise the COM port used for the USB port will not show up in the list of available ports during the install program.

    - COM Port Selection

### Changing the COM Port After Install Has Completed

If you would like to change the COM port after the installation program has completed, it’s easiest to uninstall the Palm OS Simulator and reinstall it selecting the new COM port.

# Using Palm OS Simulator With JCI Applications

## Desktop Icons and Windows Menu Items

After the setup program has completed, you will notice three new icons on you desktop.  To launch one of the JCI Palm OS applications, select the desktop icon for the application you wish to run.

    - Desktop Icons

You may also launch the JCI Palm OS applications from the Windows start program menu.  Figure 2.1.2 below shows the program menu location of the JCI Palm OS applications from the start menu.

    - Launch Menus

## Launching JCI Palm OS Applications

To run the VMA Balancing Tool or the JCI Checkout Tool, double click VBT or the Checkout shortcuts or select the application from the Start Program launch menu as shown in Figures 2.1.1 and 2.1.2.  For generating Checkout reports, double click the Checkout Report icon.  Launching the VBT or Checkout will run the Palm OS Simulator using the COM port selected during the setup program.

After the JCI Palm application has been launched, run the program just as you would from a real Palm device.  Use your mouse to select various onscreen options and functionality.  Refer to the user manual for each JCI Palm application for specific feature and user detail.   To exit the Palm OS Simulator, select the home icon on the lower left corner of the Simulator or select the Windows close box on the upper right corner of the Simulator.  Figures 2.2.1, 2.2.2, and 2.2.3 show the first screen that will appear after launching the VBT, Checkout, or Checkout Report applications.

### Launch Examples

| - VBT Launch | - Checkout Launch |
| --- | --- |
| - Checkout Report Launch |  |

# Generating JCI Wiring Checkout Reports

Wiring checkout reports can be generated by selecting Checkout Report desktop icon and selecting the Hotsync icon in the Palm OS Simulator.  This process is referred to as a Palm OS Simulator Hot Sync.

## Requirements

- Palm Desktop software installed on the same machine as the Palm OS Simulator.  This software can be downloaded for free from palmOne at : http://www.palmone.com/us/software/desktop/ .

- N2Check Conduit Software ( version 2.2 or higher ).  After installing the Palm Desktop ( previous step ), run the setup program for the N2Check Conduit.  The setup program called checkout conduit install.exe, should be included on the Palm OS Simulator install CD or on the JCI FSC website.

- A saved N2Checkout session .  See section 3.2.

- An updated palmsim.ini file.  See section 3.3.

- Hot Sync Manager configured for local synchronization.  See section 3.4.

- Palm OS Simulator Configured to Hot Sync to PC.  See section 3.5.

- Perform a Hot Sync.  See section 3.6.

## Saving A Palm OS Session

In order to run a Hot Synch to generate N2 Check reports, you must first save the current Palm OS session from within an N2 Check session.  To save a session, right click with the mouse from any place within the Palm OS Simulator screen and select Storage->Save as shown in Figure 3.2.1 below. You must do this each time you wish to update a saved session.

    - Saving Your Palm OS Session

Important::  The default session file name is storage snapshot.ssf as shown in Figure 3.2.2.  If you change this name, you must also update the StorageSnapshotFile  entry in the palmsim.ini  file ( located in the same directory as the Palm OS Simulator ).  Refer to the appendix of the document for instructions on updating the palmsim.ini file.

    - Using Default Session File Name

## Updating The Palmsim.ini file

The Palm OS Simulator uses the palmsim.ini file to store critical information regarding operation of the software.  Most important to the JCI applications is the location and file name of the Palm OS Simulator session.  This session file is loaded upon initialization of the Palm OS Simulator and contains stored data for Palm applications from previous sessions.  For the JCI Checkout tool, this session data contains the databases for user created Checkout projects.  The palmsim.ini file is located in the same directory as the Palm OS Simulator ( selected during the software installation ).  To update this file, open it using Windows notepad ( or any other text editor ) and edit accordingly.  The most likely change may be to update the stored session file name / location.

To update the session file name and location from within the palmsim.ini file, open the palm.ini file with any text editor.  In the text file, you will see two lines that that read similar to what is shown below.  Your file location may differ depending upon where you stored the session file after completing your JCI Checkout session.

LastStorageSnapshot= C:\Program Files\Johnson Controls\Palm OS Simulator\storage snapshot.ssf

StorageSnapshotFile= C:\Program Files\Johnson Controls\Palm OS Simulator\storage snapshot.ssf

Copy the LastStorageSnapshot session location to the StorageSnapshotFile so both entries have the same filename and location and save the palm.ini file.  The next time the Palm OS Simulator is run it will open with the previous session information including your saved Checkout data.  You will have to edit this entry each time you change the name or file location of a saved Palm OS session.

## Updating The Palm Hot Sync Manager

You will now have to change your Palm Desktop settings to enable communication between the Palm OS Simulator and the Palm Desktop.  If the Palm Desktop is not installed on the PC do so now.  The Palm Desktop application may be downloaded, free of charge from: http://www.palmone.com/us/software/desktop/

Verify the Palm Hot Sync Manager is running on your PC by looking for the Hot Sync Manager icon from the toolbar in the lower right corner of the screen as shown in Figure 3.4.1.  If the Hot Sync Manager is not running, it can be started from Start  Programs  PalmDesktop  HotSyncManager .  After the Palm Hot Sync Manager is running, right click on the Hot Sync Manager icon from the toolbar and check the Network option as shown in Figure 3.4.2.  Network enabled Hot Syncs ( required for syncing with the Palm OS Simulator ) will now be enabled.  You should only have to do this once.

    - Hot Sync Manager Icon Location

 Lower Left Corner of Windows Task Bar

    - Hot Sync Manager Network Selection

 Lower Left Corner of Windows Task Bar

## Configuring The Palm OS Simulator For Generating Wiring Checkout Reports

You will need to configure the Palm OS Simulator Hot Sync yourself, follow the steps indicated below.

- Select and launch the Checkout Report Hot Sync. See Figure 3.6.1

- From the HotSync application in the Palm OS Simulator, select Modem as shown in blue see below.

Hot Sync Configuration Figure 1

- Open the menu by clicking on HotSync in the upper left corner of the screen or selecting the menu icon in the lower left corner.

Hot Sync Configuration Figure 2

- Select Modem Sync Prefs from the menu.  Select Network and click OK.

- Select LANSync Prefs from the menu.  Select LANSync and click OK.

- Select Primary PC Setup from the menu and enter 127.0.0.1 as the Primary PC Address.  Select OK.

Hot Sync Configuration Figure 3

- Select Connection Setup from the menu.  Select Cradle/Cable and click Done.

Hot Sync Configuration Figure 4

- Select Service.  Change Connection to Cradle/Cable.  Select Done.

Hot Sync Configuration Figure 5

- Save this Palm OS session as described in section 3.2 in this document.   Note: Use the default session name ( storage snapshot.ssf ) unless you wish to save multiple sessions.  Note that if you change the session file name or location, you must update the palmsim.ini file to use the new session name as described in section 3.2.

Hot Sync Configuration Figure 6

## Launching the Palm Sync Application

Note:: This section assumes the Hot Sync session and Hot Sync Manager have already been configured as described in Sections 3.2 through 3.5 of this document.

Launch the Hot Synch for the Palm OS Simulator using the Checkout Report shortcut.

    - Wiring Checkout Report Generation Shortcut

After you have launched the Checkout Report shortcut, you will see the Palm OS Simulator as shown in Figure 3.4.2 below.

    - Palm OS Hot Sync Screen For Checkout Report Generation

Select the HotSync button ( center of screen ) to perform a HotSync Operation.  If this is the first Hot Synch operation you have performed, you will be prompted to enter a device name, enter any name.

    - Hot Sync Manager New User

After you select OK, the Palm OS Hot Sync will begin and a wiring checkout report will be generated for each user project defined in the wiring checkout JCI Palm application.  After the Palm OS Sync completes, you may see a warning indicating that the Hot Synch produced one or more messages.  If you open the message log, a warning log may appear as shown in Figure 3.4.4.  This message can be ignored.

    - Palm Hot Sync Warning To Ignore

Important Note !!!

After you have successfully completed you first Hot Sync, the Hot Sync Manager may generate another warning on subsequent Hot Syncs of saved sessions. This warning will state there was an error restoring the backup database and a dialog may appear with a button to reset the Palm OS simulator after the Hot Sync completes.  The Wiring Checkout reports may NOT contain your latest updates !!!  If this happens, delete the entire Hot Sync user Backup directory located in the Palm Desktop user directory as shown in Figure 3.6.5 and re-run the Hot Sync.

    - Hot Sync Backup Directory Locations

C:\Program Files\Palm ( or PalmOne )\<User Name entered during sync>\Backup

## Wiring Checkout Report Location

After the Palm Hot Sync completes, a wiring checkout report will be generated for each project defined in the wiring checkout application.  The location of these reports will be in the users directory of the Palm Desktop application.  The default location of the Palm Desktop is the Windows Program Files directory.  The Palm Desktop directory name will be Palm or PalmOne, depending upon which Palm Desktop version you have installed.

    - Sample Wiring Checkout Report Locations

C:\Program Files\Palm ( or PalmOne )\<User Name entered during sync>\Checkout

C:\Program Files\Palm\<JCIUSER\Checkout

C:\Program Files\PalmOne\<JCIUSER\Checkout

# Appendix

## Determining the COM Port

If you do not know which COM port to use for the Palm OS Simulator installation, follow these steps.  Navigate to the Control Panel by selecting System  (Start  Settings  ControlPanel  System).

From the Hardware tab, select Device Manager.

From the list of available COM ports, you can view the vendor name of the driver for the port to help determine which COM port you would like to use.

## Troubleshooting Tips

### Wiring Checkout Tool  - Cannot Run Simulator After Saving / Restoring Session

When running in Win XP platforms, the Palm OS Simulator will occasionally fail to startup after saving a session.  If this happens, open the Windows Task Manager and end all occurrences of the PalmSim_54_rel processes as shown in Figure 4.2.1 below.  The saved session should remain valid the next time you launch the Wiring Checkout tool or the Wiring Checkout Hot Sync shortcuts.

    - Ending Palm OS Simulator Process

### Wiring Checkout Tool  - Wiring Checkout Hot Sync Does Not Generate Report

The Palm OS Simulator Hot Sync, used for generating Wiring Checkout reports requires the Palm Hot Sync manager.  If the Wiring Checkout report generation fails, verify the Hot Sync manager is running.  Section 3.4 in this document explains how to start and configure the Hot Sync manager.

### Wiring Checkout Tool  - Saved Sessions Do Not Get Restored

If a saved session does not appear to load, verify that the saved session name matches the StorageSnapshotFile entry in the Palmsim.ini file.  Refer to section 3.3 in this document.

### Wiring Checkout Tool – Hot Sync Error When Generating Wiring Checkout Reports

When selecting the Hot Sync to generate Wiring Checkout reports, the Hot Sync Manager may display an error ( Figure 4.2.2 ) stating that a Service has not been selected.  If this happens, mouse click within the Select Service box ( Figure 4.2.3 ) and then tap the Done button at the bottom of the Preferences dialog.  You should now be able to perform a Hot Sync.  After the Hot Sync has completed, save the Palm OS session otherwise this error will appear each time a Hot Sync is performed.

    - Hot Sync Error – Service Not Selected

    - Fixing Hot Sync Error – Service Not Selected

| Select Service Box |  |
| --- | --- |